GS V20&GS V21

SDK USER GUIDE

Version 1.0

Nov - 12

Copyright

GENERALSCAN reserves the right to make changes to any product to improve reliability, function, or design.

GENERALSCAN does not assume any product liability arising out of, or in connection with, the application or use of any product, circuit, or application described herein.

Service Information

If you have any questions with the GSV20&GSV21, you can contact technical support email: Support@generalscan.cloud.

Revision History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | Jan-12 | Initial Release |

Overview

This document explains how to use BLE for data communication with GS V20&GS V21, including command formats and how to Bluetooth programming .

Communication

Bluetooth name

| Model | Description | Name in BLE scan |
| --- | --- | --- |
| GS V20 | Laser digital tape | GSV20-SN |
| GS V21 | Digital tape | GSV21-SN |

BLE Service

| Function | Service UUID | Characteristic UUID | Access |
| --- | --- | --- | --- |
| Data Transfer | 0000fff0-0000-1000-8000-00805f9b34fb | 0000fff1-0000-1000-8000-00805f9b34fb | Notify |
|  |  | 0000fff2-0000-1000-8000-00805f9b34fb | Write |

Digital tape transmit data to APP

We provide a testing demo Measuresync in the Measuresync SDK.zip.

The first step is to search for Bluetooth.

The second step is to choose and connect to Bluetooth.

After successful connection, press the “M” key on Digital tape, Digital tape may send data to APP flowing the Data Format.

Data Format

[Mark]<SP>[Data]<SP>[Unit]<CR><LF>

<SP> Space(0x20)

<CR> Return(0x0D)

<LF> Newline(0x0A)

| Mark | UNIT | Data Example | Description |
| --- | --- | --- | --- |
| L | mm/cm/m/ft/in | L 240 mm | Length |
| W | mm/cm/m/ft/in | W 480 mm | Width |
| H | mm/cm/m/ft/in | H 180 mm | Height |
| A | m2/ft2 | A 1.234 m2 | Area |
| V | m3/ft3 | V 2.0736 m3 | Volume |
| TP1 | mm/cm/m/ft/in | TP1 2400 mm | Triangle plus 1 |
| TP2 | mm/cm/m/ft/in | TP2 210 mm | Triangle plus 2 |
| TP3 | mm/cm/m/ft/in | TP3 2300 mm | Triangle plus 3 |
| TPL | m/ft | TPL 4.5000000 m | Triangle plus length |
| TM1 | mm/cm/m/ft/in | TM1 2400 mm | Triangle minus 1 |
| TM2 | mm/cm/m/ft/in | TM2 2100 mm | Triangle minus 2 |
| TM3 | mm/cm/m/ft/in | TM3 2000 mm | Triangle minus 3 |
| TML | m/ft | TML 0.200000 m | Triangle minus length |

*TP and TM are not functional in V21

APP transmit data to digital tape

After successful connection, you can query the information of the digital tape and send commands to set the parameters of the digital tape, APP may send data to Digital tape flowing the Query Format , Write format and Read format.

Query Format

Request:

[Command] <CR><LF>

| Status | Description |
| --- | --- |
| A | Acquired, processed |
| F | Failed |
| E | Error |

Response:

[Command] <SP> [Status]<SP>[Data]<CR><LF>

*Status E responses when request type error

| Command | Response | Description |
| --- | --- | --- |
| CT | CT A Title | Current displayed title（UTF-8） |
| CV | CV A 123 mm | Current readout |

Write format

Request:

[WRITE]<SP>[Command]<SP>[Data]<CR><LF>

Response:

[WRITE]<SP>[Status]<SP>[Command]<CR><LF>

Read format:

[READ]<SP>[Command]<SP>[Data]<CR><LF>

Response:

[READ]<SP>[Status]<SP>[Command]<SP>[Data]<CR><LF>

| Command | Response | Description |
| --- | --- | --- |
| ASLEEP_SEC | WRITE A ASLEEP_SEC | Write sleep time |
| ASLEEP_SEC | READ A ASLEEP_SEC [Data] | Read sleep time |
| AOFF_SEC | WRITE A AOFF_SEC | Write power off time |
| AOFF_SEC | READ A AOFF_SEC [Data] | Read power off time |

* Please refer to Query Format for status.

Bluetooth programming for GSV20&GSV21

Android

Scan digital tape

@Override

protected void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

...

// initialize Bluetooth adapter, get a Bluetooth adapter from the Bluetooth manager (API level must android 4.3 or higher)

final BluetoothManager bluetoothManager = (BluetoothManager) getSystemService(Context.BLUETOOTH_SERVICE);

mBluetoothAdapter = bluetoothManager.getAdapter();

...

}

// start / stop ble scan.

private void scanLeDevice(final boolean enable) {

if (enable) {

// Stops scanning after a pre-defined scan period.

mHandler.postDelayed(new Runnable() {

@Override

public void run() {

mScanning = false;

mBluetoothAdapter.stopLeScan(mLeScanCallback);

}

}, SCAN_PERIOD);

mBluetoothAdapter.startLeScan(mLeScanCallback);

} else {

mBluetoothAdapter.stopLeScan(mLeScanCallback);

}

}

// Device scan callback.

private BluetoothAdapter.LeScanCallback mLeScanCallback = new BluetoothAdapter.LeScanCallback() {

@Override

public void onLeScan(final BluetoothDevice device, int rssi, byte[] scanRecord) {

runOnUiThread(new Runnable() {

@Override

public void run() {

String name = device.getName();

String address = device.getAddress();

HashMap<String, Object> map = new HashMap<>();

map.put(DeviceName, name);

map.put(DeviceAddress, address);

_deviceList.add(map);

}

});

}

Connect digital tape

/**

* Initializes a reference to the local Bluetooth adapter.

*/

public boolean initialize() {

// For API level 18 and above, get a reference to BluetoothAdapter through

// BluetoothManager.

if (_bleManager == null) {

_bleManager = (BluetoothManager) _service.getSystemService(Context.BLUETOOTH_SERVICE);

if (_bleManager == null) {

Log.e(TAG, "Unable to initialize BluetoothManager.");

return false;

}

}

_bleAdapter = _bleManager.getAdapter();

if (_bleAdapter == null) {

Log.e(TAG, "Unable to obtain a BluetoothAdapter.");

return false;

}

return true;

}

/**

* Connects to the GATT server hosted on the Bluetooth LE device.

*/

public boolean connect() {

// Previously connected device. Try to reconnect.

if (_bleGatt != null) {

Log.d(TAG, "Trying to use an existing mBluetoothGatt for connection.");

if (_bleGatt.connect()) {

_connectionState = STATE_CONNECTING;

return true;

} else {

return false;

}

}

final BluetoothDevice device = _bleAdapter.getRemoteDevice(_bleAddress);

if (device == null) {

Log.w(TAG, "Device not found. Unable to connect.");

return false;

}

_bleGatt = device.connectGatt(_service, false, _gattCallback);

Log.d(TAG, "Trying to create a new connection.");

_connectionState = STATE_CONNECTING;

return true;

}

Send to digital tape

public void writeCharacteristic(BluetoothGattCharacteristic characteristic, byte bytes[]) {

if (_bleAdapter == null || _bleGatt == null) {

Log.w(TAG, "BluetoothAdapter not initialized");

return;

}

// less than 20 bytes each time

_sppCharacteristic.setValue(bytes);

_bleGatt.writeCharacteristic(characteristic);

}

Receive from digital tape

// Implements callback methods for GATT events that the app cares about. For example,

// connection change and services discovered.

private final BluetoothGattCallback _gattCallback = new BluetoothGattCallback() {

@Override

public void onServicesDiscovered(BluetoothGatt gatt, int status) {

// 查找 characteristic

if (status == BluetoothGatt.GATT_SUCCESS) {

for (BluetoothGattService gattService : getSupportedGattServices()) {

if (gattService.getUuid().equals(ServiceGUID)) {

List<BluetoothGattCharacteristic> gattCharacteristics = gattService.getCharacteristics();

for (BluetoothGattCharacteristic gattCharacteristic : gattCharacteristics) {

if (gattCharacteristic.getUuid().equals(SPPCharacteristicGUID)) {

_sppCharacteristic = gattCharacteristic;

setCharacteristicNotification(_sppCharacteristic, true);

}

}

}

}

if (_sppCharacteristic != null)

doNotification(ACTION_SPP_READY);

} else {

Log.w(TAG, "onServicesDiscovered received: " + status);

}

}

@Override

public void onCharacteristicChanged(BluetoothGatt gatt,

BluetoothGattCharacteristic characteristic) {

if (SPPCharacteristicGUID.equals(characteristic.getUuid())) {

byte buf[] = characteristic.getValue();

doReceived(buf);

}

}

...

};